# Design Report Blueprint for 3D Reconstruction and Phenotypic Analysis of Crop Grains Using Control-Theoretic Modeling, Estimation, and Active Sensing

## Executive summary

This report provides a rigorous design-and-analysis blueprint for building (and documenting) an end-to-end system that reconstructs crop grain geometry in 3D and extracts phenotypic traits (morphology, surface texture/color, and—optionally—internal structure and spectral/biochemical proxies) with quantified uncertainty. The overarching technical idea is to treat the entire phenotyping line—grain handling → sensing → reconstruction → trait extraction → quality control—as a dynamical system with latent states (grain pose/shape, illumination, calibration drift, conveyor/robot kinematics) and noisy observations (images, depth maps, CT projections, spectra), enabling principled use of state-space modeling, observability/identifiability analysis, Bayesian estimation and smoothing, and feedback control for active sensing and robotic handling. This aligns naturally with high-throughput seed phenotyping platforms that already integrate robotics and imaging pipelines (e.g., phenoSeeder) [1] and with recent seed-focused 3D datasets and challenges (e.g., a 3D wheat-seed reconstruction dataset/challenge with 2D/3D stations and ground truth point clouds) [2].

Key design conclusions supported by the last decade of primary literature and open datasets:

- Best “throughput per dollar” for external grain geometry typically comes from either (a) multi-view RGB + photogrammetry/visual hull methods integrated with efficient inference (including learned 3D-from-few-views), or (b) structured-light scanning where sub-0.1 mm accuracy is achievable at modest scan times. Structured-light scanning has demonstrated grain-length/width/thickness errors on the order of ~1–2% with ~10 s per grain in a reported cereal-grain system [3].

- Highest-fidelity internal + external phenotyping is achieved by X-ray micro-CT, which can reach micrometer-scale voxel sizes (e.g., 4–15 μm/pixel reported for wheat grain μCT) but with substantial acquisition time (e.g., ~30 min per grain at high resolution in one wheat grain development study) [4]. Batch scanning can materially improve throughput for certain traits (e.g., scanning many rice grains per run in chalkiness quantification) [5].

- Hybrid architectures (RGB/3D + NIR/hyperspectral) are increasingly pragmatic: an integrated wheat platform (“Seedscreener”) reports simultaneous 3D morphology + NIR spectral trait prediction, with explicit throughput and accuracy claims in a seed-by-seed pipeline [6], while large open spectral datasets for wheat kernels (including >140k kernels) support statistical modeling and ML-based inference [7].

- Control-theoretic framing yields concrete engineering benefits: formal observability checks prevent under-instrumented designs, uncertainty propagation supports defensible phenotypic error bars, MPC stabilizes throughput under constraints, and active view planning (Next-Best-View, NBV) can be formulated as information gain maximization with feedback from partial reconstructions [8].

Mermaid diagrams and tables below can be directly embedded in a formal design report.

## Objectives and scope

Objective. Design a reproducible, high-confidence pipeline that outputs (i) 3D reconstructions (point clouds/meshes/voxels) and (ii) derived phenotypic traits with uncertainties and acceptance criteria (QC), scalable from controlled lab workflows to field-deployable setups. The design should support both measurement (metrology-grade where required) and decision-making (sorting, classification, screening) under explicit latency/throughput constraints.

Scope definition. The system is decomposed into five coupled subsystems:

- Grain handling dynamics: singulation, transport (conveyor/turntable/robot), pose stabilization, and tracking (unique grain ID per sample). Robotic handling is explicitly present in seed phenotyping systems such as phenoSeeder [1] and structured-light grain scanning systems that coordinate a robot and scanner [3].

- Sensing modalities: multi-view RGB, structured light, depth sensors (stereo/ToF), CT / X-ray, micro-CT, and hyperspectral/NIR (line-scan or snapshot), including optional fusion.

- Reconstruction: SfM/MVS, visual hull/volume carving, structured-light depth fusion, CT reconstruction (FBP/iterative), and/or neural rendering (NeRF / 3D Gaussian Splatting) depending on scenario requirements [9].

- Phenotypic trait extraction: size metrics (L/W/T), volume, surface area, curvature, crease geometry, texture, color/spectral indices, and (if CT available) internal compartments/porosity/chalkiness-associated volumes [10].

- Control, estimation, and QC: state estimation for pose and calibration drift, active sensing policies (NBV), closed-loop robot/conveyor control (MPC/robust/adaptive), and statistically defensible validation.

Non-goals (unless needed): genomic association pipelines (GWAS/QTL) and breeding decision optimization; however, the system should produce outputs compatible with downstream quantitative genetics workflows.

## Background literature survey and open datasets

Seed/grain phenotyping platforms and pipelines (last decade). Robotic and integrated seed phenotyping has been exemplified by phenoSeeder, which combines pick-and-place robotics with modular sensors and outputs 2D, 3D volumetric measures, and mass/density-related traits [1]. For higher-throughput 3D reconstruction from fewer images, a recent wheat seed 3D reconstruction dataset/challenge proposes deep learning to infer point clouds from 1–3 views, reporting ~10× imaging time reduction and ~2% relative errors for volume and principal dimensions when using three images [2]. These results are especially relevant when the bottleneck is image capture time rather than inference time, which the same work notes as negligible compared to imaging [11].

Low-cost and open seed imaging tools remain significant for high throughput. The BELT + phenoSEED platform is an open-access methodology for automated seed imaging (top/side views) with explicit emphasis on calibrated color/shape traits and throughput improvement compared with manual workflows [12]. While largely 2D, it provides key lessons for production-grade acquisition (calibration, automation, sample logistics) that transfer to 3D systems.

Structured light and high-precision 3D for grains. A cereal grain structured-light system reports extracting 3D point clouds and computing length/width/thickness/surface area/volume with average dimensional errors around ~1–2% and efficiency about 9.6 s per grain, plus ML-based filled/unfilled grain discrimination [3]. This paper is valuable because it includes a robot–scanner cooperative architecture (ROS-controlled) and explicit scanner specs and accuracy statements [3].

X-ray CT / micro-CT for grain internal phenotypes. Micro-CT is increasingly used to quantify internal and external grain morphology. A wheat grain development study reports μCT-based morphometry with 4–15 μm/pixel resolution and ~30 min acquisition per grain for compartment discrimination under their setup [4]. For rice, high-resolution micro-CT has been used to quantify chalkiness volume in vivo, with discussion of throughput (multiple grains per scan under certain resolutions) and evidence that scanned seeds can germinate normally under their tested conditions [5]. These works motivate CT when the phenotype is inherently volumetric/internal (e.g., chalkiness, embryo/endosperm structure, voids).

Spectral (NIR/hyperspectral) seed phenotyping. A “Seedscreener” platform integrates RGB/3D reconstruction with NIR spectrometer-based biochemical trait prediction (protein/starch/gluten), explicitly combining exophenotype (3D geometry) and endophenotype (spectral predictors) with reported success rate and throughput (~90 grains/h) [6]. Large wheat kernel spectral datasets exist; for example a Frontiers in Plant Science study reports a dataset of >140,000 wheat kernels across varieties for classification and feature selection modeling [7]. Open repositories also distribute seed hyperspectral datasets (e.g., VNS-HW on GitHub, hyperspectral maize seed datasets on Mendeley Data) [13].

Core 3D reconstruction algorithms and benchmarks. For RGB-only multi-view 3D, modern SfM/MVS pipelines are anchored by tools such as COLMAP—based on “Structure-from-Motion Revisited” and released with an open-source pipeline [14]—and by open-source libraries such as openMVG [15]. Reconstruction quality and algorithmic choices can be benchmarked on established MVS datasets such as Tanks and Temples [16] and ETH3D [17], while seed-specific evaluation increasingly leverages dedicated datasets like the Wheat Seed 3D Reconstruction Challenge [2].

Open datasets to prioritize in a design report. The design report should explicitly list datasets for both (a) model development and (b) external reproducibility. The table below prioritizes open/official sources and seed/grain relevance.

| Dataset / resource | Modality | Size / scope | Typical tasks | Access / license notes (as reported) |
| --- | --- | --- | --- | --- |
| GrainSet (Scientific Data 2023) | High-res RGB single-kernel images + annotations incl. size & weight fields | ~352.7K images across wheat/maize/sorghum/rice; includes per-kernel metadata and category labels | Inspection, classification, segmentation; supports morphology proxies | Deposited on Figshare under CC BY 4.0 per paper; table provides per-grain-type DOIs [18] |
| Wheat Seed 3D Reconstruction dataset/challenge (ICCVW 2023) | Multi-view images + ground truth point clouds | 2964 seeds split train/val/test; per seed includes 1 color image, 36 grayscale images, high-res point cloud (~50k pts) and resampled point clouds [11] | 3D reconstruction from 1–3 views; volume and dimensional estimation | Dataset record references challenge infrastructure; split and content described in paper [2] |
| Hyperspectral wheat kernel dataset (Frontiers 2020) | NIR hyperspectroscopy | >140,000 kernels, 30 varieties | Variety classification; spectral feature learning/selection | Dataset scale described in article [7] |
| VNS-HW hyperspectral wheat dataset (GitHub) | VIS–NIR–SWIR hyperspectral (claimed 400–1700 nm range) | Released with code/model | Wheat-seed classification; spatial-spectral modeling | Release and band range described in repository [19] |
| DON-infected wheat kernel hyperspectral dataset (Dryad) | Hyperspectral reflectance of kernels + associated metadata | Research dataset with DOI | Phenomic prediction / disease-related traits | Dataset hosted on Dryad with DOI [20] |

## System architecture options and sensing/imaging models

### Architecture options with cost–throughput–accuracy tradeoffs

A design report should present multiple architectures as design points rather than a single solution, because “best” depends on phenotype targets, throughput, and deployment setting.

The table below summarizes practical options (capability-driven, not vendor-specific). Cost ranges are approximate and should be quoted as order-of-magnitude unless you have procurement quotes; publicly stated pricing examples are cited where available (e.g., micro-CT cost ranges, structured-light scanner retail pricing, commodity depth camera pricing). [21]

| Modality / architecture | What you get | Typical accuracy scale | Throughput drivers | Cost / complexity notes |
| --- | --- | --- | --- | --- |
| Multi-view RGB photogrammetry (SfM+MVS) | Dense surface geometry (+ texture) from multiple images | Often sub-mm to ~mm depending on optics, calibration, feature quality; sensitive to surface reflectance | Image count per grain, pose coverage, compute; can be accelerated with learned 3D-from-few-views [22] | Low–moderate capex (cameras + lighting + compute); highest design effort is calibration + robust automation |
| Visual hull / volume carving (silhouette-based) | Surface/voxel model robust to specularities if silhouettes are clean | Strongly depends on number of views; seed dataset paper notes high view counts (e.g., 36) yield very low systematic volume error in sphere analysis and reduce error compared to few views [11] | Controlled background + segmentation are key; turntable/robotic acquisition time dominates | Often simpler than full MVS under low texture; requires consistent segmentation |
| Structured light (projector+camera triangulation) | High-density depth/point cloud; robust on small objects | Reported single-sided accuracy on order of 0.05 mm for a specific scanner in a grain system [3] | Scan time + robot path planning; reported ~9.6 s/grain in one pipeline [3] | Moderate cost for desktop/handheld structured-light units; e.g., consumer/prosumer scanner listings show ~$4k class pricing for a model cited in such work [23] |
| Stereo depth sensors (RGB-D; e.g., RealSense-class stereo) | Depth map + RGB (lower precision than structured light for small grains unless close-range) | Vendor spec: depth accuracy <2% at 4 m, up to 90 fps [24] | Excellent for fast pose/rough shape capture; limited by spatial resolution at close range | Commodity dev kits can be a few hundred USD (example pricing in distributor listings) [25] |
| Time-of-flight (ToF) depth | Depth map; can work in lower texture cases | Accuracy depends heavily on multipath, material, range; requires characterization | High frame rates possible; best for coarse 3D in motion | Moderate; often easier than structured light but less accurate for small objects |
| X-ray CT / micro-CT | Internal + external structure; volumetric traits; tissue/void segmentation possible | Wheat μCT example: 4–15 μm/pixel, acquisition ~30 min grain [4] | Scan time dominates; can batch scan multiple grains at lower resolution for some traits [5] | High capex: public guidance suggests benchtop micro-CT often $200k–$450k, larger systems $500k–$1M [26] |
| Hyperspectral / NIR (alone or fused) | Spectral signatures; biochemical proxies; can complement 3D shape | Depends on calibration and SNR; per-band uncertainty matters | Line-scan systems depend on conveyor speed; snapshot is easier but costlier | Requires careful radiometric calibration and illumination stability [27] |

### System block diagram and pipeline flow

A design report benefits from a clear block diagram showing where estimation and control “live”.

flowchart LR
  A[Grain input: bulk sample] --> B[Singulation & ID tracking]
  B --> C[Handling dynamics: conveyor/turntable/robot]
  C --> D[Sensor suite]
  D -->|RGB multi-view| E1[SfM/MVS or visual hull]
  D -->|Structured light / depth| E2[Depth fusion / surface recon]
  D -->|X-ray CT / micro-CT| E3[Tomographic recon]
  D -->|NIR/HSI| E4[Spectral calibration & feature extraction]

  E1 --> F[Unified 3D representation: mesh/point cloud/voxel]
  E2 --> F
  E3 --> F
  E4 --> G[Spectral trait vector]

  F --> H[Phenotypic traits: size, volume, SA, curvature, texture]
  G --> H

  H --> I[QC + uncertainty + audit trail]
  I --> J[Downstream: screening, sorting, breeding analytics]

  K[State estimation & smoothing\n(KF/EKF/UKF/PF, BA)] --> C
  K --> D
  L[Feedback control\n(MPC/robust/adaptive, NBV)] --> C
  L --> D

### Sensing and imaging models

A control-theoretic design report should explicitly define forward models (how states generate measurements) and noise models (how uncertainty enters). Below are recommended canonical models for the modalities in scope.

Geometric camera model (RGB / multi-view). Use the pinhole model with intrinsics K and extrinsics R,t:

u∼K R∣t X, u=u/wv/w,

with lens distortion handled explicitly (radial/tangential) in calibration. This formulation is standard in multi-view geometry course notes and vision libraries [28]. In practice, OpenCV’s calibration pipeline (e.g., cv.calibrateCamera) provides intrinsics and distortion estimates from planar targets [29].

Radiometric image formation and noise (RGB). For quantitative intensity (texture/color/defect), model the measured pixel y as a (possibly linearized) function of scene radiance and exposure, with noise decomposed into photon (shot) noise + read noise + quantization. For machine-vision cameras, characterizing this via EMVA 1288 is recommended because it standardizes how SNR, photon transfer curves, and related performance parameters are measured and reported [30].

Stereo / depth camera measurement model. For stereo, disparity d relates to depth Z (idealized):

Z=fBd,

so depth uncertainty grows roughly with Z2 for constant disparity error—a key throughput–accuracy coupling when grains are small and you must work close. Vendor specifications can be used as constraints/priors; for example, RealSense D455-class devices specify depth accuracy <2% at 4 m and up to 90 fps operation [24]. Academic evaluations can be cited for metrological characterization and systematic errors [31].

Structured light triangulation model. Structured light measures depth by projecting known patterns and triangulating correspondences between projector and camera(s). Practical grain systems report scanner architectures based on projector + cameras and triangulation with sinusoidal gratings [3]. In a design report, model it analogously to stereo but with “projector as inverse camera,” yielding a calibrated baseline and pixel correspondence that provides depth.

CT / micro-CT forward model (Beer–Lambert + Radon transform). For X-ray CT, measurements are line integrals of attenuation μx along rays L:

I=I0 exp​−L​μs ds,  p=−ln​II0=L​μs ds,

i.e., a Radon transform of μ. Tutorials and lecture notes explicitly connect Beer–Lambert physics to Radon/filtered-backprojection reconstruction [32]. This is central for identifiability: you can only reconstruct details that are supported by sufficient angular sampling and SNR.

Hyperspectral radiometric model. Hyperspectral pipelines should distinguish raw DN → radiance → reflectance. Calibration must address illumination, sensor response, and stray light; NIST guidance emphasizes the importance of radiometric characterization artifacts and handling scattered light in hyperspectral imagers [33]. Practical calibration procedures using low-cost reflectance references are reported in the literature [34], and industry explanations clarify the difference between raw/radiance/reflectance products [35].

## State-space modeling, observability/identifiability, and estimation/filtering

This section is the “control theory spine” of the design report: define state, measurement, and parameter models for the imaging pipeline and grain handling; analyze what is theoretically recoverable; then propose estimators with explicit assumptions.

### Dynamic models for grain handling and imaging

A minimal discrete-time grain pose model (conveyor or turntable). Let the grain’s rigid pose be represented by position pk∈R3 and orientation qk (unit quaternion), with velocities vk, ωk. Define state:

xk=pkvkqkωk.

A simple kinematic propagation (with sampling Δt) is:

pk+1=pk+Δt vk+wkp, vk+1=vk+wkv,

qk+1=qk⊗exp​Δt2ωk+wkq, ωk+1=ωk+wkω,

with process noise terms capturing slip, bouncing, and actuator uncertainty. If turntable angular velocity is commanded, ωk can be input-driven with uncertainty.

Dynamic imaging pipeline model (calibration drift + illumination). Many “mysterious” failure modes (texture loss, inconsistent color traits, depth bias drift) can be modeled as slow states:

zk=θkcalθkillumθkfocus,  zk+1=zk+wkz,

where θcal are extrinsic parameters (multi-camera, camera–projector, camera–robot hand-eye), θillum encodes illumination intensity/spectrum, and θfocus can capture defocus changes. Such “drift states” are directly motivated by industrial imaging practice and standardized sensor characterization frameworks such as EMVA 1288 [30].

Measurement models. Examples: - For RGB keypoints/contours ykrgb, measurement function hrgbxk,zk,ϕ uses the calibrated projection model; ϕ are static shape parameters (below). - For structured light depth yksl or stereo depth ykd, hd⋅ encodes triangulation geometry plus a depth noise model (often range-dependent) [36]. - For CT, yct=Apose μ+v in discretized form, where A is the ray transform (Radon system matrix) [37].

### Shape parameterization as a system identification problem

A grain is typically static during a scan, so its shape is a parameter (or a high-dimensional latent) rather than a dynamic state. The design report should justify the shape representation because it determines identifiability and compute cost.

Option A: Low-dimensional parametric models (fast, interpretable).
Represent the grain as (possibly truncated) superquadric, ellipsoid + crease perturbation, or PCA basis learned from a training set:

sα=s+Uα, α∈Rr, r≪n.

This enables classical identifiability tools because parameters are finite-dimensional.

Option B: Discrete surface/volume representations (high fidelity).
Point cloud {Xi}, mesh vertices, or voxel occupancy. Wheat seed reconstruction work explicitly uses point cloud outputs and derives traits afterward (a “white-box” approach) [11], while micro-CT studies segment compartments in voxel volumes and compute morphometric features [4].

Option C: Neural implicit representations (compact, differentiable).
NeRF represents a scene with an MLP mapping x,y,z,θ,ϕ→σ,c and uses differentiable volume rendering for novel view synthesis [38]. Instant-NGP accelerates training via multiresolution hash encodings [39]. 3D Gaussian Splatting provides explicit point-based radiance representations enabling real-time rendering [40]. These methods are increasingly relevant where (i) you want fast view synthesis and (ii) you can exploit differentiability for NBV/active sensing.

### Observability and identifiability analyses

A design report targeted to an engineering researcher should explicitly answer: which quantities can be recovered from the chosen sensors and motion?

Local observability (pose + drift). For a nonlinear system:

xk+1=fxk,uk+wk, yk=hxk+vk,

local observability can be assessed by linearization around a trajectory and checking rank of the discrete-time observability matrix:

O=CCACA2⋮CAn−1, A=∂f∂xx, C=∂h∂xx.

For grain scanning, this analysis guides minimum view count and motion excitation. A practical example: the wheat seed dataset paper explicitly notes that reconstruction quality (and systematic volume error) depends strongly on number of views in silhouette/volume carving [11]; from an observability viewpoint, insufficient viewpoints create unobservable components of the 3D shape (hidden concavities, ambiguous silhouettes).

Parameter identifiability (shape + calibration). Treat parameters θ (shape basis coefficients, intrinsics, extrinsics) as unknown constants. A standard route is examining the Fisher Information Matrix (FIM) under a likelihood model:

Iθ=k​∂hk∂θ⊤Rk−1∂hk∂θ,

where Rk is measurement noise covariance. A near-singular I indicates non-identifiability or severe ill-conditioning, signaling the need for (i) richer motions (more diverse viewpoints), (ii) better calibration targets, (iii) higher SNR, or (iv) a lower-dimensional shape parameterization.

CT identifiability. In CT, identifiability is governed by angular sampling and physics; the reconstruction problem is inversion of a Radon transform / line-integral measurements [37]. Inadequate angular coverage yields structured artifacts (“missing wedge”)—a design report should tie these artifacts to the forward model and sampling plan.

### Estimation and filtering methods with equations and assumptions

Linear Kalman filter (KF). For linear Gaussian systems:

xk+1=Axk+Buk+wk, wk∼N0,Q

yk=Cxk+vk, vk∼N0,R,

the KF provides optimal (MMSE) recursive estimation; its foundations trace to Kalman’s 1960 paper [41].

Extended Kalman filter (EKF). For nonlinear f,h, EKF linearizes. With Jacobians Fk=∂f/∂x, Hk=∂h/∂x, a standard discrete EKF uses:

xk|k−1=fxk−1|k−1,uk−1, Pk|k−1=FkPk−1|k−1Fk⊤+Q.

Kk=Pk|k−1Hk⊤HkPk|k−1Hk⊤+R−1,

xk|k=xk|k−1+Kkyk−hxk|k−1, Pk|k=I−KkHkPk|k−1.

EKF is widely taught via standard lecture notes; key limitations (approximation/divergence risk) should be stated explicitly [42].

Unscented Kalman filter (UKF). UKF avoids Jacobians by propagating sigma points through nonlinear dynamics. “Unscented Filtering and Nonlinear Estimation” provides a canonical reference and explains why UKF can outperform EKF in nonlinear regimes [43]. This is attractive for grain pose estimation when camera measurements are highly nonlinear and you want robustness without careful Jacobian engineering.

Particle filters (PF). For strongly non-Gaussian posteriors (occlusions, ambiguous silhouettes), represent pxk|y1:k by weighted particles {xki,wki}. The classic “tutorial on particle filters” is a standard reference for sequential importance sampling/resampling variants [44]. PFs are useful when measurements are multi-modal (e.g., silhouette ambiguities) or when contact dynamics cause non-Gaussian motion.

Smoothing / Moving Horizon Estimation (MHE). For offline or quasi-online refinement, solve a windowed MAP problem:

minxk−N:k∥xk−N−xk−N∥P−12+i=k−Nk−1∥xi+1−fxi,ui∥Q−12+i=k−Nk∥yi−hxi∥R−12.

MHE pairs naturally with MPC (dual estimation/control), and is discussed in MPC treatments [45].

Bundle adjustment (BA) as MAP estimation. In multi-view RGB, BA refines camera poses and 3D structure by minimizing reprojection error; “Bundle Adjustment — A Modern Synthesis” remains a canonical survey [46]. A typical objective:

min{Rj,tj},{Xi}i,j∈E​ρ​∥πK,Rj,tj,Xi−uij∥Σij−12,

where ρ is a robust loss. Production implementations often use nonlinear least squares libraries such as Ceres Solver [47].

## Control designs for active sensing and robotic handling with stability sketches

A design report that “uses modern control theory” should not stop at estimation; it must specify how sensing/handling decisions are made, and why feedback policies are stable and robust under uncertainty.

### Active sensing and Next-Best-View as optimal control / POMDP

Information-gain NBV formulation. Active volumetric reconstruction can be framed as selecting the viewpoint uk that maximizes expected information gain (or minimizes expected posterior uncertainty) given current reconstruction state. An ICRA reference explicitly formulates NBV as information gain selection using feedback from partial reconstructions [48]. In a seed/grain scanner, action uk could be next robot pose or next turntable angle subset, constrained by cycle time.

A common surrogate objective is to maximize expected reduction in entropy (or trace/det of covariance) of the shape/occupancy map:

uk⋆=argmaxu∈U EΔH∣u,current map or argminu tr Pk+1shapeu.

This marries directly with KF/UKF covariance propagation.

RL/POMDP perspective. In more complex occlusion regimes (e.g., grains in clutter, imperfect singulation), active vision can be treated as a POMDP, with learned policies. Recent surveys discuss active vision and NBV strategy taxonomies, indicating maturation from handcrafted criteria to learning-driven methods [49].

### Motion control and scheduling: MPC, robust control, adaptive control

Model Predictive Control (MPC). MPC solves constrained finite-horizon optimal control repeatedly:

minu0:N−1i=0N−1ℓxi,ui+VfxN s.t. xi+1=fxi,ui, xi∈X, ui∈U.

A comprehensive reference with stability treatment is Rawlings, Mayne, and Diehl’s MPC text [50].

Stability sketch for MPC (nominal). If you choose a terminal cost Vfx=x⊤Px and a terminal set Xf such that: - Vf is a Lyapunov function for a local stabilizing controller u=κx on Xf, - and the terminal constraint enforces xN∈Xf, then the receding-horizon cost is non-increasing and closed-loop stability follows (standard MPC Lyapunov argument) [45].

In a grain phenotyping line, MPC is most appropriate for: - robot motion planning under collision/velocity limits, - conveyor speed regulation to meet exposure constraints, - scheduling multi-sensor acquisitions to satisfy latency constraints.

Robust control (small gain / H∞). When plant/model mismatch is significant (e.g., slip variability, illumination drift), robust stability can be justified via the small gain theorem. Lecture materials on robust control explicitly cover H∞ norms, robust stability claims, and small gain theorem proofs [51]. In report form, you can present a sketch:

If the uncertainty block Δ has induced gain ∥Δ∥∞≤1, and the nominal closed-loop transfer from uncertainty input to output has gain ∥M∥∞<1, then the interconnection is robustly stable (by small gain). [51]

Adaptive control (MRAC) for handling variability. Adaptive control is useful when system parameters vary across grains/samples (friction, mass) and you can measure tracking error. MRAC design via Lyapunov arguments is covered in adaptive control lecture notes [52] and in applied stability-proof references (e.g., predictor-based MRAC with Lyapunov stability analysis) [53]. In grain handling, adaptive control can stabilize pick-and-place or turntable speed under load changes without hand-tuning every scenario.

### LQR/LQG as baseline stabilizers (and why they matter here)

For linearized handling dynamics x=Ax+Bu, the continuous-time LQR minimizes:

J=0∞x⊤Qx+u⊤Ru dt

and yields u⋆=−Kx with K derived from the Algebraic Riccati Equation. Standard LQR lecture notes provide the Riccati-based solution and conditions for stability (stabilizability/detectability) [54]. LQR/LQG serve as: - inner-loop stabilizers for robot joints or conveyor motion, - baseline controllers when MPC is too heavy for embedded hardware.

## Calibration, registration, reconstruction/traits, validation, implementation, risks, and recommended designs

This final section consolidates methods that a design report must specify to be credible, reproducible, and practically deployable.

### Calibration, registration, and data fusion methods

Camera calibration. Use planar target calibration (checkerboard/charuco/AprilTag grids). OpenCV’s calibration pipeline is an accessible standard reference for deriving intrinsics and distortion [29]. For robust fiducials and repeatable calibration under partial visibility, AprilTag/AprilGrid ecosystems are widely used in robotics [55].

Multi-sensor / temporal calibration. If you include IMUs (e.g., moving camera rigs) or multiple cameras with non-overlapping FOV, Kalibr provides batch optimization for multi-camera and camera–IMU extrinsic and temporal calibration [56].

Robot-to-sensor hand-eye calibration. For eye-on-hand systems, solve AX=XB. The Tsai–Lenz method is a classical reference; modern analyses and comparisons still use this formulation [57].

Point cloud registration. Use a two-stage strategy: global alignment + local refinement. - Global: RANSAC-based alignment or certifiable robust registration (e.g., TEASER/TEASER++ for outlier-heavy correspondences) [58]. - Local: ICP variants for refinement (implemented in Open3D and PCL) [59].

Optimization backends. Use established solvers for nonlinear least squares and factor graphs: - Ceres Solver for BA and general nonlinear least squares [47]. - g2o for graph optimization formulations (pose graphs, multiway registration) [60].

### Algorithms for 3D reconstruction and phenotypic trait extraction

Classical 3D reconstruction pipelines. - SfM/MVS (RGB): COLMAP provides a full incremental SfM pipeline and is widely used as a baseline [14]. Your design report should specify feature extraction/ matching strategy, camera model assumptions, BA schedule, and dense MVS settings. - Structured light reconstruction: explicitly document pattern strategy, triangulation calibration, depth fusion, and denoising. Grain systems demonstrate a full pipeline: plane fitting (RANSAC), region growing, voxel downsampling, and 3D trait computation [3]. - CT reconstruction: specify FBP vs iterative reconstruction; filtered backprojection is directly derived from Radon inversion in standard CT lectures [37].

Learned reconstruction and segmentation. - 3D-from-few-views for seeds: Wheat seed reconstruction work uses deep models to infer 3D point clouds from 1–3 images with reported ~2% relative errors for volume and primary dimensions using 3 images, enabling real-time sorting when imaging dominates cycle time [11]. - 2D segmentation: U-Net remains a standard architecture for precise pixel-wise segmentation [61]; Mask R-CNN provides instance segmentation with explicit masks [62]; SAM introduces promptable segmentation with a massive dataset, enabling flexible bootstrapping of labels and semi-automatic annotation in specialized domains [63]. - 3D point cloud learning: PointNet++ is a canonical architecture for hierarchical feature learning on point sets, useful for 3D trait regression or part segmentation when point clouds are the native data type [64].

Trait extraction: recommended “phenotype vector.” For each grain, a robust phenotype record typically includes: - Scale and size: length L, width W, thickness T (often via PCA axes or bounding box in canonical pose), plus aspect ratios. - Volume and surface area: V, A, convex hull volume, solidity V/Vhull. - Shape descriptors: curvature distributions, sphericity, ellipticity, crease geometry (notably in wheat; μCT work explicitly quantifies crease shape as a morphometric feature) [4]. - Surface appearance: calibrated color in a stable color space (e.g., Lab), texture statistics (GLCM or learned embeddings). - Spectral indices / features: reflectance-derived features, band ratios, or learned spectral embeddings, with careful calibration to reflectance [65]. - Internal traits (CT): compartment volumes (embryo/endosperm/voids), porosity, chalkiness volume; rice micro-CT chalkiness work motivates volume-based internal quantification [5].

### Performance metrics, experimental validation protocols, and benchmarking

A rigorous design report should include both metrology metrics (accuracy vs ground truth) and operational metrics (throughput, latency, uptime).

3D reconstruction metrics. - Point cloud distance: Chamfer distance (mean nearest-neighbor distance) and/or Hausdorff distance; Hausdorff distance is explicitly used in wheat 3D reconstruction evaluation in related work [66]. - Volumetric overlap: 3D IoU for voxel occupancy; seed reconstruction work references 3D IoU and Chamfer-type losses in 3D reconstruction contexts [11]. - Dimensional errors: relative error (%) of L,W,T,V; structured-light grain work reports such errors against manual baselines [3].

Spectral model metrics. - Regression: R2, RMSE, MAE; classification: accuracy, macro-F1. Large NIR wheat kernel studies emphasize feature selection and classification performance on large datasets [7].

Operational metrics. - Throughput: grains/hour; e.g., Seedscreener reports ~90 grains/h [6], while a structured-light method reports ~9.6 s/grain (~375 grains/h) [3]. - Latency: end-to-end per grain (ingest → reconstruction → traits). - Yield: percentage of grains successfully processed; Seedscreener reports success rate of single-kernel phenotypic data collection [6]. - Recalibration interval: drift tolerance.

Validation protocol recommendations.
In report form, specify: 1. Calibration validation (reprojection error distributions; AprilTag/chessboard repeatability; hand-eye validation via loop closure). 2. Ground truth strategy for geometry: micro-CT (highest), structured light (high), precision gauge blocks or known 3D artifacts. 3. Repeatability tests: repeated scans of the same grain under controlled conditions; compute within-grain variance and Allan-variance-like drift checks for sensors with thermal drift. 4. Cross-condition generalization: vary moisture content, cultivar, surface reflectance (glossy vs matte), dust, and lighting; document failure modes.

### Implementation considerations: real-time constraints, hardware/software stack, compute

Software stack (open and defensible). - Vision and calibration: OpenCV [29]. - SfM/MVS: COLMAP (baseline) [14]; openMVG as an alternative [67]. - Point cloud + mesh processing and registration: Open3D and/or PCL [68]. - Optimization: Ceres Solver [69]; g2o [70]. - Robotics middleware: ROS 2 (DDS-based) for modular sensing/control; ROS 2 documentation emphasizes DDS middleware choices [71] and includes guidance for real-time programming constraints [72].

Compute sizing and latency budgeting.
A design report should include a latency budget per stage (acquisition, preprocessing, reconstruction, traits, QC). This is particularly important because some pipelines are acquisition-bound (e.g., multi-view imaging time dominates; seed reconstruction from few images is motivated explicitly by acquisition time constraints) [11], while others are compute-bound (dense MVS, high-res CT reconstruction). Record GPU requirements for deep models and note memory demands for volumetric CT.

### Risk analysis and mitigation

A credible design report should enumerate failure modes and mitigations; below is a concise, engineering-focused set.

Optical failures (RGB / structured light). Specular reflections, low contrast, and motion blur can collapse correspondence quality. Silhouette-based methods (visual hull/volume carving) are explicitly noted as robust under challenging seed surface properties (specular, low contrast) in the wheat seed reconstruction literature [11], but they shift the burden to segmentation quality and view coverage.

Calibration drift and domain shift. Illumination spectrum drift or camera thermal drift can bias color/spectral traits. Use EMVA 1288-informed characterization for camera noise/SNR stability [30], radiometric calibration routines for hyperspectral [27], and incorporate slow drift into a state estimator (θkillum) with periodic recalibration triggers.

CT-specific risks. X-ray dose and scan time are major constraints. Rice chalkiness micro-CT work explicitly discusses in vivo scanning and checks germination impacts for their regime [5], but a design report should still include radiation safety compliance and throughput limits cited for micro-CT cost and time [73].

Data and ML risks. Label scarcity and domain shift can harm segmentation/regression. Mitigate with: - open datasets for pretraining (GrainSet for 2D kernels) [18], - semi-automatic labeling (SAM-assisted) [63], - explicit evaluation on held-out cultivars/years.

### Recommended design choices for three use-case scenarios

The table below gives recommended architectures (not the only feasible ones), aligned with the engineering trade space and cited performance/cost anchors.

| Scenario | Recommended sensing stack | Reconstruction + estimation | Expected accuracy (order-of-magnitude) | Expected throughput (order-of-magnitude) | Cost class (capex) | Rationale |
| --- | --- | --- | --- | --- | --- | --- |
| Low-cost, high-throughput screening (lab/seed facility) | Multi-view RGB (few views) + controlled lighting; optional commodity stereo depth | Visual hull / learned 3D-from-few-views; EKF/UKF for pose tracking; periodic BA on calibration set | Dimensional/volume errors ~few % when using few views (seed dataset reports ~2% for key traits with 3 images) [11] | Hundreds–thousands grains/day depending on automation; acquisition is short | Low–moderate (cameras, lighting, motion stages); can use low-cost depth cameras with distributor pricing examples [74] | Designed for throughput; leverages seed-specific dataset evidence that “3 images is enough” for many traits [11] |
| High-accuracy lab phenomics (internal structure + external geometry) | Micro-CT (and/or structured light for external) + calibrated color imaging | CT reconstruction (FBP/iterative) via Beer–Lambert/Radon model; segmentation + morphometry; uncertainty quantification | Micrometer-scale voxel sizes reported (4–15 μm/pixel) enabling compartment/crease/internal void traits [10] | Low unless batch scanning; individual grain scans can be ~tens of minutes [4] | Very high: micro-CT cost guidance suggests ~$200k–$450k benchtop; up to $1M+ for larger [26] | Necessary when phenotype is internal (chalkiness, embryo/endosperm structure) and you need micrometric resolution [75] |
| Field-deployable or near-field portable phenotyping | Single/few-view RGB + depth (stereo RGB-D) + optional portable NIR | Robust 3D proxies + learned estimators; NBV-lite (limited motion) for stability; compressive inference on edge GPU | Coarser 3D accuracy; prioritize repeatability over absolute micrometric accuracy | High if acquisition is quick; compute constrained | Moderate; relies on commodity sensors and embedded compute; RealSense-class devices provide 90 fps and <2% @ 4 m specs [24] | Enables deployment outside lab; sacrifices internal traits unless using radiographic methods; emphasizes robust control of lighting and motion |

### A practical development and validation timeline

A design report often benefits from an explicit timeline that ties control/estimation deliverables to hardware milestones.

gantt
  title Development and validation timeline for grain 3D phenotyping system
  dateFormat  YYYY-MM-DD
  axisFormat  %b %Y

  section Requirements and risk
  Define phenotypes, throughput targets, ground truth plan  :a1, 2026-03-17, 30d
  Sensor trade study + preliminary observability analysis   :a2, after a1, 30d

  section Prototype build
  Build handling + sensing rig (v1)                         :b1, after a2, 45d
  Calibration toolchain (camera, hand-eye, spectral)        :b2, after b1, 30d

  section Algorithms and estimation
  Baseline reconstruction + trait extraction                :c1, after b2, 45d
  State estimator (EKF/UKF) + QC uncertainty propagation    :c2, after c1, 30d
  Active sensing / MPC scheduling (if robotic views)        :c3, after c2, 45d

  section Validation and benchmarking
  Ground-truth study (structured light / micro-CT subset)   :d1, after c1, 60d
  Benchmark on open datasets + cross-condition tests        :d2, after d1, 45d

  section Deployment hardening
  Latency optimization + logging + reproducibility package  :e1, after d2, 45d

### Implementation notes for writing the design report itself

To make the report “rigorous and audit-ready,” explicitly include:

- A model appendix stating f⋅, h⋅, noise assumptions (Gaussian vs non-Gaussian), and calibration priors, referencing standardized sensor characterization where applicable (EMVA 1288) [30].

- A reproducibility appendix listing datasets used and licenses (e.g., GrainSet CC BY 4.0, dataset sizes, DOIs) [18].

- A control appendix defining stability assumptions (stabilizability/detectability), and using MPC/Lyapunov arguments grounded in standard MPC references [50].

- A software bill of materials (OpenCV, COLMAP/openMVG, Open3D/PCL, Ceres/g2o, ROS2), citing official documentation [76].

[1] https://academic.oup.com/plphys/article/172/3/1358/6115817

https://academic.oup.com/plphys/article/172/3/1358/6115817

[2] [11] [22] https://openaccess.thecvf.com/content/ICCV2023W/CVPPA/papers/Cherepashkin_Deep_Learning_Based_3d_Reconstruction_for_Phenotyping_of_Wheat_Seeds_ICCVW_2023_paper.pdf

https://openaccess.thecvf.com/content/ICCV2023W/CVPPA/papers/Cherepashkin_Deep_Learning_Based_3d_Reconstruction_for_Phenotyping_of_Wheat_Seeds_ICCVW_2023_paper.pdf

[3] https://www.nature.com/articles/s41598-022-07221-4.pdf

https://www.nature.com/articles/s41598-022-07221-4.pdf

[4] [10] https://link.springer.com/content/pdf/10.1186/s13007-019-0468-y.pdf

https://link.springer.com/content/pdf/10.1186/s13007-019-0468-y.pdf

[5] [75] https://link.springer.com/article/10.1186/s12284-020-00429-w

https://link.springer.com/article/10.1186/s12284-020-00429-w

[6] https://www.sciencedirect.com/science/article/pii/S0168169923007664

https://www.sciencedirect.com/science/article/pii/S0168169923007664

[7] https://www.frontiersin.org/journals/plant-science/articles/10.3389/fpls.2020.575810/full

https://www.frontiersin.org/journals/plant-science/articles/10.3389/fpls.2020.575810/full

[8] [48] https://rpg.ifi.uzh.ch/docs/ICRA16_Isler.pdf

https://rpg.ifi.uzh.ch/docs/ICRA16_Isler.pdf

[9] [14] https://demuc.de/papers/schoenberger2016sfm.pdf

https://demuc.de/papers/schoenberger2016sfm.pdf

[12] https://link.springer.com/article/10.1186/s13007-020-00591-8

https://link.springer.com/article/10.1186/s13007-020-00591-8

[13] [19] https://github.com/RSMagneto/SSANet

https://github.com/RSMagneto/SSANet

[15] https://link.springer.com/chapter/10.1007/978-3-319-56414-2_5

https://link.springer.com/chapter/10.1007/978-3-319-56414-2_5

[16] https://vladlen.info/papers/tanks-and-temples.pdf

https://vladlen.info/papers/tanks-and-temples.pdf

[17] https://openaccess.thecvf.com/content_cvpr_2017/papers/Schops_A_Multi-View_Stereo_CVPR_2017_paper.pdf

https://openaccess.thecvf.com/content_cvpr_2017/papers/Schops_A_Multi-View_Stereo_CVPR_2017_paper.pdf

[18] https://www.nature.com/articles/s41597-023-02660-8.pdf

https://www.nature.com/articles/s41597-023-02660-8.pdf

[20] https://datadryad.org/dataset/doi%3A10.5061/dryad.d2547d8bx

https://datadryad.org/dataset/doi%3A10.5061/dryad.d2547d8bx

[21] [26] [73] https://www.microphotonics.com/how-much-does-a-micro-ct-scanner-cost/

https://www.microphotonics.com/how-much-does-a-micro-ct-scanner-cost/

[23] https://m.madeinchina.com/mall/show-Wiiboox-Reeyee-PRO-Full-Color-Handheld-3D-Scanner_990066.html

https://m.madeinchina.com/mall/show-Wiiboox-Reeyee-PRO-Full-Color-Handheld-3D-Scanner_990066.html

[24] [36] https://www.realsenseai.com/products/real-sense-depth-camera-d455f/

https://www.realsenseai.com/products/real-sense-depth-camera-d455f/

[25] [74] https://www.digikey.com/en/product-highlight/i/intel/d455-realsense-depth-cameras

https://www.digikey.com/en/product-highlight/i/intel/d455-realsense-depth-cameras

[27] [33] https://tsapps.nist.gov/publication/get_pdf.cfm?pub_id=903023

https://tsapps.nist.gov/publication/get_pdf.cfm?pub_id=903023

[28] https://web.stanford.edu/class/cs231a/course_notes/01-camera-models.pdf

https://web.stanford.edu/class/cs231a/course_notes/01-camera-models.pdf

[29] [76] https://docs.opencv.org/4.x/dc/dbb/tutorial_py_calibration.html

https://docs.opencv.org/4.x/dc/dbb/tutorial_py_calibration.html

[30] https://www.emva.org/standards-technology/emva-1288/

https://www.emva.org/standards-technology/emva-1288/

[31] https://www.mdpi.com/1424-8220/21/22/7770

https://www.mdpi.com/1424-8220/21/22/7770

[32] https://tristanvanleeuwen.github.io/IP_and_Im_Lectures/tomography.html

https://tristanvanleeuwen.github.io/IP_and_Im_Lectures/tomography.html

[34] [65] https://www.mdpi.com/1424-8220/21/11/3738

https://www.mdpi.com/1424-8220/21/11/3738

[35] https://resonon.com/blog-raw-radiance-reflectance-hyperspectral-data

https://resonon.com/blog-raw-radiance-reflectance-hyperspectral-data

[37] https://courses.healthtech.dtu.dk/22485/notes/ct_reconstruction_ver_2025_2_slides.pdf

https://courses.healthtech.dtu.dk/22485/notes/ct_reconstruction_ver_2025_2_slides.pdf

[38] https://arxiv.org/abs/2003.08934

https://arxiv.org/abs/2003.08934

[39] https://arxiv.org/abs/2201.05989

https://arxiv.org/abs/2201.05989

[40] https://arxiv.org/abs/2308.04079

https://arxiv.org/abs/2308.04079

[41] https://www.cs.unc.edu/~welch/kalman/media/pdf/Kalman1960.pdf

https://www.cs.unc.edu/~welch/kalman/media/pdf/Kalman1960.pdf

[42] https://web.stanford.edu/class/ee363/lectures/ekf.pdf

https://web.stanford.edu/class/ee363/lectures/ekf.pdf

[43] https://www.cs.ubc.ca/~murphyk/Papers/Julier_Uhlmann_mar04.pdf

https://www.cs.ubc.ca/~murphyk/Papers/Julier_Uhlmann_mar04.pdf

[44] https://people.eecs.berkeley.edu/~pabbeel/cs287-fa12/optreadings/Arulampalam_etal_2002.pdf

https://people.eecs.berkeley.edu/~pabbeel/cs287-fa12/optreadings/Arulampalam_etal_2002.pdf

[45] [50] https://sites.engineering.ucsb.edu/~jbraw/mpc/MPC-book-2nd-edition-4th-printing.pdf

https://sites.engineering.ucsb.edu/~jbraw/mpc/MPC-book-2nd-edition-4th-printing.pdf

[46] https://www.cs.jhu.edu/~misha/ReadingSeminar/Papers/Triggs00.pdf

https://www.cs.jhu.edu/~misha/ReadingSeminar/Papers/Triggs00.pdf

[47] [69] https://ceres-solver.org/nnls_tutorial.html

https://ceres-solver.org/nnls_tutorial.html

[49] https://www.sciencedirect.com/science/article/pii/S2667393225000171

https://www.sciencedirect.com/science/article/pii/S2667393225000171

[51] https://archive.control.lth.se/media/Education/DoctorateProgram/2018/Robust%20Control/Lecture3.pdf

https://archive.control.lth.se/media/Education/DoctorateProgram/2018/Robust%20Control/Lecture3.pdf

[52] https://courses.ece.ubc.ca/574/EECE574-11.pdf

https://courses.ece.ubc.ca/574/EECE574-11.pdf

[53] https://ntrs.nasa.gov/api/citations/20090032037/downloads/20090032037.pdf

https://ntrs.nasa.gov/api/citations/20090032037/downloads/20090032037.pdf

[54] https://engineering.purdue.edu/~byao/Courses/ME578%20-%20Yao%20-%20Spring%202018/Chap9_overhead_filled.pdf

https://engineering.purdue.edu/~byao/Courses/ME578%20-%20Yao%20-%20Spring%202018/Chap9_overhead_filled.pdf

[55] https://april.eecs.umich.edu/software/apriltag

https://april.eecs.umich.edu/software/apriltag

[56] https://github.com/ethz-asl/kalibr

https://github.com/ethz-asl/kalibr

[57] https://kmlee.gatech.edu/me6406/handeye.pdf

https://kmlee.gatech.edu/me6406/handeye.pdf

[58] https://arxiv.org/abs/2001.07715

https://arxiv.org/abs/2001.07715

[59] https://www.open3d.org/docs/release/tutorial/pipelines/icp_registration.html

https://www.open3d.org/docs/release/tutorial/pipelines/icp_registration.html

[60] [70] https://ais.informatik.uni-freiburg.de/publications/papers/kuemmerle11icra.pdf

https://ais.informatik.uni-freiburg.de/publications/papers/kuemmerle11icra.pdf

[61] https://arxiv.org/abs/1505.04597

https://arxiv.org/abs/1505.04597

[62] https://arxiv.org/abs/1703.06870

https://arxiv.org/abs/1703.06870

[63] https://arxiv.org/abs/2304.02643

https://arxiv.org/abs/2304.02643

[64] https://arxiv.org/abs/1706.02413

https://arxiv.org/abs/1706.02413

[66] https://www.mdpi.com/2077-0472/14/3/391

https://www.mdpi.com/2077-0472/14/3/391

[67] https://openmvg.readthedocs.io/en/latest/

https://openmvg.readthedocs.io/en/latest/

[68] https://www.open3d.org/html/

https://www.open3d.org/html/

[71] https://ros2docs.robook.org/humble/Installation/DDS-Implementations.html

https://ros2docs.robook.org/humble/Installation/DDS-Implementations.html

[72] https://ftp.udx.icscoe.jp/ros/ros_docs_mirror/en/humble/Tutorials/Real-Time-Programming.html

https://ftp.udx.icscoe.jp/ros/ros_docs_mirror/en/humble/Tutorials/Real-Time-Programming.html